METRIX MX5060 Multimeter

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for METRIX MX5060 Multimeter.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

METRIX MX5060 Multimeter

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

Class/Panel Name:                    Function Name:

Initialize                        mx5060_init

Configure

Voltage                        mx5060_voltage

Current                        mx5060_current

Frequency                      mx5060_frequency

Resistance                     mx5060_resistance

Capacitance                    mx5060_capacitance

Temperature                    mx5060_temperature

Mode

Filter                         mx5060_filter

Measurement                       mx5060_measure

Error                             mx5060_error

Close                             mx5060_close

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

mx5060_capacitance

int mx5060_capacitance (int range, int _2ndDisplay);

Purpose

Switches the instrument to Capacitance measurement.

Parameter List

range

Variable Type       int

Specifies the range of the measurement.

Variable Type:  integer

Variable Range:  0 -  Auto

1 -  6 nF

2 -  60 nF

3 -  600 nF

4 -  6 F

5 -  60 F

6 -  600 F

7 -  6 mF

8 -  60 mF

Default Value:   0 - Auto Range

_2ndDisplay

Variable Type       int

Specifies the second type of measurement.

Variable Type: integer

Valid Range:   0 -  None

1 -  MAX

2 -  MIN

5 -  delta MEM + delta REL

Default Value: 0 -  None

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mx5060_close

int mx5060_close (void);

Purpose

Closes the instrument.

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mx5060_current

int mx5060_current (int function, int range, int _2ndDisplay,

int lowPass);

Purpose

Switches the instrument to current measurement.

Parameter List

function

Variable Type       int

Specifies the type of measurement.

Variable Type: integer

Valid Range:   0 -  DC

1 -  AC

2 -  AC+DC

Default Value: 0 -  DC

range

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range:  0 - Auto

1 - 6000 A

2 - 60 mA

3 - 600 mA

4 - 6 A

5 - 10 A

Default Value:   0 - Auto

_2ndDisplay

Variable Type       int

Specifies the second type of measurement.

Variable Type: integer

Valid Range:   0 -  Hz

1 -  MAX

2 -  MIN

3 -  Peak+

4 -  Peak-

5 -  delta MEM + delta REL

Default Value: 0 -  Hz

lowPass

Variable Type       int

Specifies the lowPass of measurement.

Variable Type: integer

Valid Range:   0 - Off

1 - On

Default Value: 0 -  Off

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mx5060_error

int mx5060_error (char errorMessage[]);

Purpose

Returns the code error

Parameter List

errorMessage

Variable Type       char []

Specifies the buffer to write the error message.

Variable Type:  char *

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mx5060_filter

int mx5060_filter (int filter);

Purpose

Changes the Filter of the instrument.

Parameter List

filter

Variable Type       int

Specifies the rejection of the instrument

Variable Type:  integer

Variable Range:  1 - On

0 - Off

Default Value:   0 - Off

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mx5060_frequency

int mx5060_frequency (int _2ndDisplay);

Purpose

Sets the instrument to frequency measurement.

Parameter List

_2ndDisplay

Variable Type       int

Specifies the second type of measurement.

Variable Type: integer

Valid Range:   0 -  None

1 -  MAX

2 -  MIN

5 -  delta MEM + delta REL

Default Value: 0 -  None

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mx5060_init

int mx5060_init (int portCom, int IDQuery, int resetDevice);

Purpose

Panel Name:    Initialize

Description:   Initializes the instrument in the

following ways:

- opens the instrument

- optionally performs an

identification query on the

instrument

- optionally sets the instrument

to a default configuration

Parameter List

portCom

Variable Type       int

Control Name:  Port_Com

Description:   Specifies the number of serial port to

communicate with the instrument.

Variable Type: Integer

IDQuery

Variable Type       int

Control Name:  ID Query

Description:   Specifies if an ID Query is sent

to the instrument during the

initialization procedure.

Variable Type: Integer

Valid Range:   1 = Yes

0 = No

Default Value: 1 - Yes

resetDevice

Variable Type       int

Control Name:  Reset

Description:   Specifies if the instrument is to

be reset during the initialization

procedure.

Variable Type: Integer

Valid Range:   0 = No Reset

1 = Reset device

Default Value: 1 = Reset

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mx5060_measure

int mx5060_measure (double *measure, char type[]);

Purpose

Returns the measurements of the instrument.

Parameter List

measure

Variable Type       double (passed by reference)

Displays the results of the measurement in

Scientific Format.

Variable Type:  double

type

Variable Type       char []

Specifies the buffer to write the unit of the measurement.

Variable Type:  char *

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mx5060_resistance

int mx5060_resistance (int measure, int range, int _2ndDisplay);

Purpose

Sets the instrument to Resistance measurement.

Parameter List

measure

Variable Type       int

Specifies the measurement

Variable Type:  integer

Variable Range:  0 -  Resistance

1 -  Continuity testing

2 -  Diode testing

Default Value:   0 -  Resistance

range

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range:  0 -  Auto

1 -  600  Ohm

2 -  6 kOhm

3 -  60 kOhm

4 -  600 kOhm

5 -  6 MOhm

6 -  60 MOhm

Default Value:   0 - Auto

_2ndDisplay

Variable Type       int

Specifies the second type of measurement.

Variable Type: integer

Valid Range:   0 -  None

1 -  MAX

2 -  MIN

5 -  delta MEM + delta REL

Default Value: 0 -  None

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mx5060_temperature

int mx5060_temperature (int unit, int range);

Purpose

Sets the instrument to Temperature measurement.

Parameter List

unit

Variable Type       int

Specifies the unit of temperature.

Variable Type:  integer

Variable Range:  0 - C

1 - F

Default Value:   0 - C

range

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range:  0 -  Auto

1 -  200C or 400F

2 -  1200C or 2200F

Default Value:   0 - Auto

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4

________________________________________________________________________________

mx5060_voltage

int mx5060_voltage (int function, int range, int _2ndDisplay,

int lowPass);

Purpose

Switches the instrument to voltage measurement.

Parameter List

function

Variable Type       int

Specifies the type of measurement.

Variable Type: integer

Valid Range:   0 -  DC

1 -  AC

2 -  AC+DC

3 -  VlowZ

Default Value: 0 -  DC

range

Variable Type       int

Specifies the range of the measurement

Variable Type:  integer

Variable Range:  0 - Auto

1 - 60 mV

2 - 600 mV

3 - 6 V

4 - 60 V

5 - 600 V

6 - 1000 V

Default Value:   0 - Auto

_2ndDisplay

Variable Type       int

Specifies the second type of measurement.

Variable Type: integer

Valid Range:   0 -  Hz

1 -  MAX

2 -  MIN

3 -  Peak+

4 -  Peak-

5 -  delta MEM + delta REL

Default Value: 0 -  Hz

lowPass

Variable Type       int

Specifies the lowPass of measurement.

Variable Type: integer

Valid Range:   0 - Off

1 - On

Default Value: 0 -  Off

Return Value

Control Name:  Error

Description:   Displays the results of the

function call.

Variable Type: Integer

Errors:

0   Success

-1   Unknown system error

-2   Invalid port number

-3   Port is not open

-4   Unknown IO error

-5   Unexpected internal error

-6   No serial port found

-7   Cannot open port

-10   Settings conflict

-11   Memory allocation error

-12   Unable to allocate system resources

-13   Invalid parameter

-14   Invalid baud rate

-99   I/O operation timed out

-101   Error of parameter 1

-102   Error of parameter 2

-103   Error of parameter 3

-104   Error of parameter 4